# Section 10999

Miscellaneous Specialties

# PART 1 GENERAL

1.01	Section Includes

- Tower Clock Components and Remote Control System.

- Related Sections

## Basic Materials and Methods: All raceways, junction boxes, and conduit wire to be provided by electrical contractor.

# PART 2 PRODUCTS

2.01	CLOCK

A.	Furnish complete clock system consisting of (n) tower clock with (1) remote control system.

“n” is the quanitity of clocks required

B.	Manufacturer:  Electric Time Company, Inc., 97 West Street, Medfield, MA, USA 1-(508) 359-4396/800-531-2562 FAX 1-(508) 359-4482 - http://www.electrictime.com - salese@electrictime.com.

C.	Tower Clock Components

- Finishes: Dial markings and clock hands to be made of aluminum with a polyurethane painted finish, color to be specified from manufacturer’s standards.

- Hands:  As shown on architectural drawing, to be halo lit with white LED’s.

- Dial Markings: As shown on architectural drawing, to be halo-lit with white LED’s.

- Movement: Style MI Design. 24VAC drive motor and electronics. 8 second maximum run time. Alternating hall effect switched closed loop minute impulse operation.  Composite lubrication free bearings.

- Slip ring and brush assembly to bring power to clock hands.

- Single source: Clock, movement and controller to be manufactured by the same company.

- LED drivers to be located behind the wall.

- Style: WP-1000-MI-I Design,   outer diameter as shown on architetural drawings, rear access, LED halo lit hands and markers.

# D.	Provide Type 99B-MI automatic reset control;

- IP65 Enclosure – Indoor & Outdoor Use

- Automatically resets clock after power failures.

- Automatically resets clocks for daylight savings time

- Precision Quartz Time base 4 minutes per year maximum drift

- Built in 100 year daylight savings time calendar.

- 2 Line 16 character backlighted LCD display.

- ETL listed to UL 863.

- Power Failure Event logging.

- Standard MI output – 4 clocks maximum standard

- Standard 24VDC RP output -20 clocks maximum

- Sweep Second hand output

- Hour Strike Capability.

- RS-232 and RS-485 output ports

- 24 VAC hour strike output – configurable pulse output

# PART 3 EXECUTION

3.01	Installation

- Inspect substrates, supporting items and related conditions, to ensure that they are ready to receive each item, prior to commencement of installation.

B.	Install each item at locations indicated on drawings, as detailed, and in accordance with manufacturers’ instructions.