# Canister Design Tower Clock

# PART 1 GENERAL

1.01	Section Includes

A.	1.	Tower Clock Components and Remote Control System.

B.	Related Sections

- Basic Materials and Methods: All raceways, junction boxes, conduit wire to be provided by electrical contractor.

# PART 2 PRODUCTS

2.01	CLOCKS

A.	Furnish complete clock system consisting of (N) tower clock and (1) remote control system.

## Please specify quantity of clock “N”

B.	Manufacturer:  Electric Time Company, Inc., 97 West Street, Medfield, MA, USA (508)-359-4396/(800)-531-2562 FAX (508)-359-4482 – http://www.electrictime.com – sales@electrictime.com

C.	Tower Clock Components

Dial:

Single sheet (not pieced) translucent milky-white acrylic.

Finishes:

Dial markings to have a matte black finish, case, bezel and hands to have a black polyurethane finish.

Hands:

As selected by architect.

Dial Markings:

As selected by architect.

Illumination:

Backlit with white LEDs and their associated LED drivers.

Movement:

Style MI Design. 24VAC drive motor and electronics. 8 second maximum run time. Alternating hall effect switched closed loop minute impulse operation.  Composite lubrication free bearing. Single source: Clock and movement to be manufactured by the same company.

Listing:

Provide UL 48 listing on clock assembly

Crystal:

Provide clear, flat tempered glass crystal.

Case:

Aluminum case and bezel.

Style:	Style 66XX

## Please specify size of clock “XX”

## (15”, 24”, 30”, 36”, 42”, 48”).

Important when semi-flush mounting ring required add the following;

Ring:

Provide semi-flush  mounting ring.

D.	Provide Type 99B-MI automatic reset control;

- IP65 Enclosure – Indoor & Outdoor Use

- Automatically resets clock after power failures.

- Automatically resets clocks for daylight savings time (if required)

- Precision Quartz Time base 4 minutes per year maximum drift.

- Optional GPS – no drift.

- Built in 100 year daylight savings time calendar.

- 2 Line 16 character backlighted LCD display.

- ETL listed to UL 863.

- Power Failure Event logging.

- Standard MI output – 4 clocks maximum standard

- Standard 24VDC RP output -20 clocks maximum

- Sweep Second hand output

- Hour Strike Capability.

- RS-232 and RS-485 output ports

- 24 VAC hour strike output – configurable pulse output

E. Lighting Control

Provide CTRL-ELEC-PHOTO-T15 photoelectric cell to control the illumination.

##### PART 3 EXECUTION

3.01	Installation

A.	Inspect substrates, supporting items and related conditions, to ensure that they are ready to receive each item, prior to commencement of installation.

B.	Install each item at locations indicated on drawings, as detailed, and in accordance with manufacturers’ instructions.